import json

import numpy as np

import time

import paho.mqtt.client as mqtt

import csv

from datetime import datetime

import os

from collections import deque

# --- CONFIGURATION ---

MQTT_BROKER_IP = "localhost"

MQTT_BROKER_PORT = 1883

DATA_TOPIC = "uwb/tag/data"

STATUS_TOPIC = "uwb/tag/status"

RESULT_TOPIC = "uwb/tag/result"

# --- File Path ---

LOG_FILE_NAME = 'position_log.csv'

# --- 1. CALIBRATION ---

ANCHOR_BIAS = {

"1782": 0.0,

"1783": 0.0,

"1781": 0.0

}

# --- 2. GEOMETRY CONFIG ---

ANCHOR_CONFIG = {

"1782": [-0.204, 0.0],

"1783": [0.204, 0.0],

"1781": [0.0, 0.59]

}

# --- 3. PHYSICS LIMITS ---

MIN_DIST = 0.01

MAX_DIST = 50.0

TAG_HEIGHT = 0.7

MAX_SPEED_MPS = 3.0

ROOM_BOUNDS = { "x_min": -5.0, "x_max": 5.0, "y_min": -5.0, "y_max": 5.0 }

# --- 4. KNOWN START POSITION ---

# We use this to initialize the filter

TRUE_X = 0.0

TRUE_Y = 1.5

# -------------------------------

# --- 5. SMOOTHING SETTINGS ---

HISTORY_LENGTH = 10

# --- MINI KALMAN FILTER CLASS ---

class MiniKalmanFilter:

def __init__(self):

# --- CALIBRATION CHANGE: Initialize at Known Start ---

# State: [x, y, vx, vy]

# We start at the known position with 0 velocity

self.x = np.array([TRUE_X, TRUE_Y, 0.0, 0.0])

# Initial Uncertainty (P)

# We are very confident about position (low variance),

# but uncertain about velocity (high variance).

self.P = np.diag([0.1, 0.1, 5.0, 5.0])

# -----------------------------------------------------

self.Q = np.eye(4) * 0.0001

self.R = np.eye(2) * (0.3**2)

self.H = np.array([[1, 0, 0, 0], [0, 1, 0, 0]])

self.dt = 0.1

def predict(self):

F = np.array([[1, 0, self.dt, 0], [0, 1, 0, self.dt], [0, 0, 1, 0], [0, 0, 0, 1]])

self.x = np.dot(F, self.x)

self.P = np.dot(np.dot(F, self.P), F.T) + self.Q

def update(self, z):

z = np.array(z)

y = z - np.dot(self.H, self.x)

S = np.dot(np.dot(self.H, self.P), self.H.T) + self.R

K = np.dot(np.dot(self.P, self.H.T), np.linalg.inv(S))

self.x = self.x + np.dot(K, y)

self.P = np.dot((np.eye(4) - np.dot(K, self.H)), self.P)

def trilaterate_2d(anchors, distances):

anchors = np.array(anchors); distances = np.array(distances)

try:

p1 = anchors[0]; r1 = distances[0]

A, b = [], []

for i in range(1, len(anchors)):

pi = anchors[i]; ri = distances[i]

A.append(2 * (pi - p1))

b.append(r1**2 - ri**2 - np.dot(p1, p1) + np.dot(pi, pi))

pos, *_ = np.linalg.lstsq(np.array(A), np.array(b), rcond=None)

return np.array([pos[0], pos[1]])

except: return None

def on_connect(client, userdata, flags, rc):

print(f"Connected to MQTT Broker (Code: {rc})")

client.subscribe(DATA_TOPIC, qos=1)

client.subscribe(STATUS_TOPIC, qos=1)

def is_physically_possible(new_pos, last_pos, dt):

x, y = new_pos

if not (ROOM_BOUNDS["x_min"] <= x <= ROOM_BOUNDS["x_max"]):

print(f"  ⚠️ REJECT: Out of bounds ({x:.2f}, {y:.2f})")

return False

if not (ROOM_BOUNDS["y_min"] <= y <= ROOM_BOUNDS["y_max"]):

print(f"  ⚠️ REJECT: Out of bounds ({x:.2f}, {y:.2f})")

return False

distance_moved = np.linalg.norm(new_pos - last_pos)

speed = distance_moved / dt

if speed > MAX_SPEED_MPS:

print(f"  ⚠️ REJECT: Speed violation ({speed:.2f} m/s)")

return False

return True

def on_message(client, userdata, msg):

if msg.topic == STATUS_TOPIC:

try:

status = msg.payload.decode('utf-8')

print(f"\n== STATUS UPDATE: Tag is now {status} ==\n")

except: pass

return

try:

payload_str = msg.payload.decode('UTF-8')

uwb_data = json.loads(payload_str)

uwb_list = uwb_data.get("links", [])

log_file_handle = client.log_file_handle

log_csv_writer = client.log_csv_writer

kf = client.kalman_filter

history = client.pos_history

current_time = time.time()

last_time = getattr(client, 'last_update_time', current_time - 0.5)

client.last_update_time = current_time

dt = current_time - last_time

kf.dt = dt if dt > 0 else 0.1

except Exception: return

current_anchors, current_distances = [], []

for link in uwb_list:

try:

addr = link.get("A"); raw_dist = float(link.get("R"))

if addr in ANCHOR_CONFIG:

bias = ANCHOR_BIAS.get(addr, 0.0)

d_raw = raw_dist - bias

if d_raw > TAG_HEIGHT:

d_horizontal = np.sqrt(d_raw**2 - TAG_HEIGHT**2)

if MIN_DIST < d_horizontal < MAX_DIST:

current_anchors.append(ANCHOR_CONFIG[addr])

current_distances.append(d_horizontal)

else:

current_anchors.append(ANCHOR_CONFIG[addr])

current_distances.append(0.01)

except Exception: pass

kf.predict()

is_good_measurement = False

tag_est = None

if len(current_anchors) >= 3:

candidate_pos = trilaterate_2d(current_anchors, current_distances)

if candidate_pos is not None:

last_x, last_y = float(kf.x[0]), float(kf.x[1])

if is_physically_possible(candidate_pos, np.array([last_x, last_y]), dt):

is_good_measurement = True

tag_est = candidate_pos

kf.update(z=tag_est)

else:

pass

x_filt_raw = float(kf.x[0])

y_filt_raw = float(kf.x[1])

history.append((x_filt_raw, y_filt_raw))

final_x = sum(p[0] for p in history) / len(history)

final_y = sum(p[1] for p in history) / len(history)

angle_rad = np.arctan2(final_x, final_y)

angle_deg = np.degrees(angle_rad)

dist_origin = np.sqrt(final_x**2 + final_y**2)

error_m = np.sqrt((final_x - TRUE_X)**2 + (final_y - TRUE_Y)**2)

print(f"  Pos: ({final_x:.2f}, {final_y:.2f}) | Angle: {angle_deg:.1f}° | Error: {error_m:.2f}m")

try:

result_payload = json.dumps({

"x": final_x, "y": final_y, "angle": angle_deg, "dist": dist_origin, "error": error_m

})

client.publish(RESULT_TOPIC, result_payload)

except: pass

try:

timestamp = datetime.now().isoformat()

calc_x, calc_y = (tag_est[0], tag_est[1]) if is_good_measurement else (np.nan, np.nan)

log_data = [timestamp, calc_x, calc_y, final_x, final_y, angle_deg, dist_origin, TRUE_X, TRUE_Y, error_m]

log_csv_writer.writerow(log_data)

log_file_handle.flush()

except Exception: pass

def main():

print("\n>>> RUNNING SERVER v14 (Calibrated Start) <<<\n")

print(f"Filter Initialized at: ({TRUE_X}, {TRUE_Y})")

file_needs_header = not os.path.exists(LOG_FILE_NAME)

try:

log_file = open(LOG_FILE_NAME, 'a', newline='')

csv_writer = csv.writer(log_file)

if file_needs_header:

csv_writer.writerow(["timestamp", "x_calc", "y_calc", "x_filtered", "y_filtered", "angle_deg", "dist_origin", "x_true", "y_true", "error_m"])

log_file.flush()

except Exception as e: print(f"FATAL: {e}"); return

client = mqtt.Client()

client.kalman_filter = MiniKalmanFilter()

client.pos_history = deque(maxlen=HISTORY_LENGTH)

client.log_file_handle = log_file

client.log_csv_writer = csv_writer

client.last_update_time = time.time()

client.on_connect = on_connect

client.on_message = on_message

try: client.connect(MQTT_BROKER_IP, MQTT_BROKER_PORT, 60)

except Exception as e: print(f"Connection Error: {e}"); log_file.close(); return

client.loop_start()

print("Network loop started. Waiting for UWB data...")

try:

while True: time.sleep(1)

except KeyboardInterrupt:

client.loop_stop(); client.disconnect(); log_file.close()

print("Server stopped.")

if __name__ == '__main__':

main()